Exercise 2 Addendum

# After completed exercise two

# A) Explore the dec0corel.vbe file (BOP output file)

B) Explore the file dec0core.vst file:

- How many models are used?

- How many instances of these models are used?

- How many internal signals interconnect the cells?

- Draw a schematic representation of the dec2to4.vst file

# C) Explore the chip.vst file

- How many models are used?

- How many instances of these models are used?

- How many internal signals interconnect the cells?

- Draw a schematic representation of the dec2to4.vst file.

# D) Explore the layout

- Which area occupies the core of the chip

- Which area occupies the chip

- How many metal layers do you identify?

- How do the metal layers contact each other?

- How do a metal layer make contact with diffusion areas?

- How do a metal layer make contact with a transistor gate?

- Are all gates of the same length and/or width? Why?.

# E) Write a true table for a 3 to 8 bits decoder and write the logic equations to describe it.

F) Write a true table for a 4 to 2 bit encoder and write a set of logic equations to describe it. What about non legal input?

G) Write a true table for an 8 to 3 bit encoder and write the logic equations to describe it.